# Lane Mine – Bonanza Silver-Gold Vein System

Yavapai County, Arizona

Lane Mine is a historic high-grade silver–gold vein system in the Bradshaw Mountains of Arizona, about 4.5 miles south of Crown King.

The project hosts three parallel quartz veins within an ~18-meter structural corridor, yet only the center vein was mined historically. The hanging-wall and footwall veins remain largely unexplored.

## Bonanza Grades

Historic sampling returned exceptional silver values:

- ~5.8 g/t gold

- ~31,000 g/t silver

Additional sampling returned:

- ~8,040 g/t silver

Historic shipping ore reportedly exceeded:

- ~6,850 g/t silver

These are bonanza-grade polymetallic vein values rarely seen today.

## Historic Development

Work in the 1880s included:

- Shaft development

- ~66 m drift tunnel

- Stoping on the center vein

A later exploration tunnel was driven ~122 m to intersect the orebody ~62 m below the historic workings, but work stopped before reaching the vein, leaving the downward extension of the ore shoot untested.

## Geological Setting

- Hosted in muscovite–biotite granite

- Veins strike N50°E

- Dip ~75° west

- Mineralization includes galena and sphalerite carrying silver, gold, lead, and zinc in quartz veins.

Vein widths range from:

- 10–20 cm high-grade streaks

- up to ~1.2 m ore zones

## Why It’s Interesting

Three major upside drivers:

- Only one of three veins was mined. Two parallel veins remain essentially untouched.

- The high-grade shoot is untested at depth. A lower tunnel stopped short of the orebody.

- Exceptional silver grades. Values typical of historic bonanza silver camps.

## Bottom Line

Lane Mine is a high-grade silver-gold vein system with extreme silver values, multiple parallel structures, and minimal historic development.

The opportunity is to test:

- two unmined veins, and

- the down-dip extension of the historic ore shoot.